WOODWORK MARKING SCHEME

1. 	Features common to all saws

- Size is determined by length and function of blade

- Cut by taking out small pieces in the form of dust

- Fineness/coarseness of cut determined by number of points

- All have teeth i.e projections ending in thin top

- The teeth have to be sharpened and shaped periodically to be efficient

- Handles made of wood or plastic

- Blade is made of high carbon steel

2.	Differentiation of wood carving and wood turning

  - Wood carving is the art of shaping wood using hand tools

  - Wood turning is the art of shaping wood using machine

3.	Safety precautions

(i)	Carring hand tools

- Do not carry many tools at once

- Do not carry sharp edged or pointed tools in pocket

- When carrying sharp edged or pointed tools,  holding them close to the body and pointed down to floor

- When passing sharp edge tools to fellow worker  pass the handle first

(ii)	When storing

- Returning the tool to their correct places after use

- Always wipe free of grease or dirt after use

- Sharp point to face inside cabinet or cabinet or downwards when hung on shelves

4.	Effects of using unseasoned timber

- Strength properties will be reduced

- Fungal attack is more likely

- There is excessive weight due to the amount of water present

- The surface finish is unsuitable to receive either paint or polish

- Adhesives will have minimal holding properties

- Corrosive properties increases

- Timber will shrink when drying

- The wood is unworkable by hand tools

5.  (a) Functions of roots

- anchor the tree to the ground

- extract water and minerals from the ground

2x1-2 marks

(b)	Functions of the Irunk

- produces timber

- acts as passage of water and minerals from the roots to the leaves

- supports the crown stores food

Any 2x1-2 marks

6. Pictorial sketches of Double and Twin Tenon

Correct tenon			1

Pictorial sketch			1

2 marks x 2-4 marks

7. (a) Advantages of using a roller when painting

- It is faster to paint

- It covers a wider area

- It is ideal for wide boards

(b) Sketch of vencer hammer

8. To avoid splining of wood when boring

Method 1

- Bore until the screw tip shows on the back side of the workpiece

- Remove bit and bore from the other side

Method 2

- Clamp the work piece with a backing of a waste piece

- Bore the hole till bit depth is greater than work piece thickness

2x2-4 marks

9. (a) Precautions required for steel tape

Before starting work check its accuracy with a vernier calliper or new steel rule

- To ensure the hook at the end is not bent/loose

- Do not pull ur push the tape when the locking device is engaged.

- Do not extend the tape beyond its limit

- Do not drop the tape measure

(b) Factors influencing choice of a finish

- appearance protection

- durability

- case of application safety

- reversibility

- hygiene

Any 4 x ½ =2 marks

10. Reduction of scale

SECTION B

12. (a) Characteristics of lacquer

- Very quick drying time

- Ease of application with spray equipment

- Ease of repair and removal

- Relatively low cost

- Great versatility for colour matching

- Wide range of possible formulations for specific situations

- Excellent rubbing qualities

- Exceptional film clarity, producing the appearance of great depth

- Large variety of thinner solvent blends

Any 6 x ½ =3 marks

(b) Use of a test strip

- Make a test a strip using a scrap of the same wood as that to be stained

- Paint the test strip with one coat of stain and allow to dry

- Paint the strip with a second coat but leave a small section of the first coat exposed for comparison

- If required, make three or four coats and leave them to dry thoroughly.

- Apply a band of clear finish along one half of the test strip to see how it affects

- cach coat of stain

(c)  Types of spokeshave

- Flat spokeshave

- It has a flat sole and straight iron

- It is designed for planning in either a straight line or on a convex surface

- Round spokeshave It has a round sole and concave iron

- It is designed to cut concave curves in timber

Name 2x½=1

Description 2 x 1-2

3 marks)

13.(a) Cutting action of cross-cut saw teeth

When cutting across the grain the teeth of the saw must be shaped in a manner that they can sever the fibres on each side of the saw kerf. For this to happen, adjacent teeth are brought to points on opposite sides of the blade. The points and knife-like edges on the teeth cut the fibres forming a kerf. The teeth are set to provide clearance for the blade in the kerf.  The front of the teeth are inclined at about 70-80 so that they do not burry themselves in the fibre.

Any 6 x 1-6 marks

(b)	 Methods of matching veers

(i) 	Book matching

A book match uses successive leaves of veneer

Every other leaf is turned over like the pages of a book, and edge-joined in this manner.

The reverse side of one leaf is the mirror image of the succeeding leaf. The result is a series of pairs

(ii) Slip matching

This match is used to create a wide veneer from narrow ones

Consecutive vencers are slipped sideways and edge-joined together without altering their grain direction. This provides pattern repetition.

(iii) Diamond matching

The sheets are cut at an angle and quarter-matched to produce a diamond figure. The sheets are matched with a common centre, joined side by side.

14. (a)  (i) Tool is a ratchet brace (1)

(ii) Functions

W-Knob	-	used to apply pressure using the palm (1)

X-Bearing	-	to facilitate easy movement of the handle in use (1)

Y-Chuck		-	Houses the jaws and provides adjustment of jaws (1)

Z-Jaw		-	provides grip to the shank of drill bits (1)

Name of tool			=	1 mark

Naming 4 parts@1 		=	4 marks

Functions of the 4 parts@	=	4.marks

9 marks

(b)	Areas of study in woodwork

- Engineering

- Furniture design and production

- Wood science

- Pulp and paper Production of timber and related wood products

- manufacturing

- Forestry

Accept any other related area

Any 4 x 1-4 marks

(c)	Mistakes to avoid when giving first aid treatment for single burns

- Do not apply grease, ointment or medication to the burn

- Do not remove any clothing that sticks to a burn

- Do not break blisters

- Do not use dry cotton or material with loose fibre to cover the burn

- Do not touch the burned area

15.(a)