Name………………………………………..ADM No……………. Class………………..

448/1

ELECTRICITY

MARKING SCHEME

Paper 1

(THEORY)

AUGUST-SEPTEMBER 2022

Time: 2 ½ hours

MECS CLUSTER JOINT EXAMINATION

FORM FOUR END YEAR  EXAMINATION  2022

ELECTRICITY

INSTRUCTIONS TO CANDIDATES

- Write your name, admission number and class in the spaces provided

- Each candidate should have the following:

- Drawing paper,A4

- Drawing instruments

- Electronic calculator

3. This paper has two sections; A and B

4. Answer ALL the questions in section A and ANY FOUR in section B

5. All measurements are in millimeters unless it is stated otherwise .

This paper consists of 11 printed pages, a certain that all pages are printed and no     questions are missing

SECTION A (48marks)

Answer all the questions in this section in the spaces provided

- (a)	List four insulating materials used in electrical installations.			(2 marks)

-PVC

-bakelite

-Polychloroprene

-rubber

(b)	State two advantages of PVC sheathed cables over tough rubber  sheathed.														                      (2 marks)

It is the cheapest system

It is easy and quick to install

P.v.c. is impervious to many chemicals and free from ageing

- (a)	State Lenz’s law of electromagnetic induction.					(2 marks)

The direction of induced emf is such that itopposes the change that causes it

(b)	Name four applications of electromagnets.						(2 marks)

DC machines

AC machines

Measuring instruments

Moving coil loud-speaker

- (a)	Name four National Polytechnics in Kenya.						(2 marks)

Eldoret, kabete, Kisumu, Meru, Nyeri

(b)	List two business opportunities in the field of electricity.				(1 mark)

Electrical/ Electronics parts seller

Electronics repair shop

Electronics waste disposal

Cyber café shop

- (a)	State how each of the following electrical material waste should be disposed safely:

- Burnt fluorescent tubes;							(½ mark)

Take to hazardous waste collection centre

- Damaged computers.								(½ mark)

Donated to community as second or third grade computers

(b)	State the application of each of the following types of extinguishers:

- Carbon dioxide;								(1 mark)

Extinguish electric fires

- Water.

Extinguish solid fuel and fluid fuel fires					(1 mark)

- (a)	Figure 1 shows a carbon resistor with color codes. Determine the value of the resistor given,													(2 marks)

1 0 x 102  ±5%

1KΩ

- Figure 2 shows a resistive circuit.

Determine the:

- Value of the resistor B if the total circuit resistance is 20Ω;				(4 marks)

10+12xB12+B=20

2B=120

B=60Ω

- Total circuit current.									(2 marks)

I=VsRt     2020=1A

- (a)	Name four parts of a fluorescent fitting.						(2 marks)

Starter

Electrodes

Tube

choke

- List four marking out tools used in fabricating sheet metal casing.			(2 marks)

Scriber

Steel rule

dividers

try square

centre punch

- (a)	Figure 3 shows two views of an object drawn in third angle projection.

On the grid provided, make a free hand isometric sketch with corner N as the lowest point.													(3 marks)

(b)	Outline two ways that can be used to troubleshoot a faulty television.		(2 marks)

Test working of fuse plug

Test capacitor at the input

- (a)	With the aid of sketches, distinguish between P-N-P and an N-P-N transistor.	(3 marks)

(b)	Name four applications of a P-N junction diode.					(2 marks)

Power rectification

In smoothing circuits

In photo diodes

In light emitting diodes

In zener diodes

- (a)	State three advantages of digital instruments over analogue instruments.		(3 marks)

Easy to read

More accurate

Less expensive

(b)	An ideal transformer connected to a 240V mains supplies a 12V, 120W lamp. Calculate the:

- Transformer’s turns ratio;							(3 marks)

V1V2 =24012=20

- Current taken from the supply.						(3 marks)

V1V2=I2I1      I1=12120x10 =1A

(c) With the aid of diagram describe 'armature reaction' in d.c motors              (3mks)

This is the distortion and weakening of the main magnetic field in a dc machine due the field’s reaction with the armature’s magnetic flux.

Normal axis

Shifted Neutral axis

Direction of rotation

N

S

SECTION B (52 marks)

Answer any four questions in the spaces provided.

- (a)	(i)	Convert 2310 to binary.							(2 marks)

1112

(ii)	Convert 110112 to decimal.							(2 marks)

2710

(b)	state seven types of  logic gate;

(4 marks)

NOT gate, OR gate ,AND gate,NOR gate, the NAND gate, the XOR gate and the XNOR gate.

(ii)	Draw a truth table for  the; NAND.									                                                                                                                                     (4 marks)

| A | B | F |
| --- | --- | --- |
| 0 0 1 1 | 0 1 0 1 | 1 1 1 0 |

(c)	Name two applications of logic gates.						(1 mark)                                    In  burglar alarms

As switches

For computation analysis in  circuits

- (a)	Draw a sine wave and indicate the following:

- Peak value;									(5 marks)

- Instantaneous value;

- Cycle.

Alternation

Vpp

Vm

Cycle

(b) Figure 4 shows an RLC circuit. The frequency is 50Hz

Calculate the:

- Inductive reactance;									(2 marks)

XL=2πfL=2πx50x0.15=47.1Ω

.

- Capacitive reactance;									(2 marks)

Xc=12πfc=12πx50x100µF 31.831Ω

- Circuit impedance;									(2 marks)

Z=(R2+XL-XC144+233.14=377.14Ω

- Circuit current.									(2 marks)

I=VsZ=200377.14=0.530A

- (a)	State;

- Two advantages of a moving coil instrument.				(2 marks)

It is shielded from magnetic fields

Inexpensive compared to moving iron meters

Have uniforn scale

- Two essential features of an analogue instrument.				(2 marks)

Controlling feature

Damping feature

Deflecting feature

(b)	With the aid of a labelled diagram, explain the Flemmings right hand rule to demonstrate the		direction of induced EMF.								(6 marks)

N

S

Coil on former

Battery

I

Magnetic

Flux

(c)	show that for two capacitors C1 and C2 connected in series, the total capacitance is given by:

CT=C1×C2C1+C2

(3 marks)

1Ct=1C1+1C2=C1+C2C1C2=Ct=c1c2/c1+c2

- (a)	State:

- Two advantages of trunking over steel conduit wiring systems.		(2 marks)

cables are laid in rather than pulled in.

can take large number of cables

- Two advantages of MCBs over rewirable fuses.				(2 marks)

Can be reset after rectifying fault

Faulty circuit is indicated by MCB

Can be used as a switch

(b)	(i)	Draw and label a diagram of a switch start fluorescent fitting.		(6 marks)

Fluorescent coating

Iron cored inductor

A.C. supply

Glass tube containing low-pressure mercury vapour and argon gas

Oxide coated lamp electrode

Fluorescent coating

P.f. Correction capacitor

Glow starter with R.I.S. capacitor

L

N

(ii) 	Outline three tests carried out in a completed electrical installation.	(3 marks)

Earthing tests

Ring circuit continuity test

Insulation resistance tests

Verification of polarity tests

- Figure 5 shows a machine block drawing of an isometric projection.

Draw the following views in full size in first angle projection:

- Front elevation in the direction of arrow F:

- Plan;

- Indicate six dimensions.								(13 marks)

Solution-3mks mks

Accuracy-3mks

Neatness-2mks

Dimensions-4mks

Borderline-1mk

THIS IS THE LAST PRINTED PAGE