TERM 1 – 2026

HOMESCIENCE

CLOTHING CONSTRUCTION

PRACTICAL PAPER 2 (441/2)

FORM FOUR (4)

Time: 2½ Hours

MARKING SCHEME

|  |  | MAXIMUM SCORE | ACTUAL SCORE |
| --- | --- | --- | --- |
| 1. (a) (b) (c) (d) | PRESENTATION Work well pressed (1) and folded (½) Label (½) firmly fixed (1) without concealing details (½) and on a single fabric (½) Pins/needles (½) unnecessary tacking threads (½) and loose threads (½) removed Made up for the right half | 1½ 2½ 1½ 1 |  |
|  |  | 6½ |  |
| 2. (a) (b) (c) (d) (e) (f) (g) (h) | CUTTING OUT All seven pieces cut out (7 x ½) Smooth cutting of yoke C.F (1) and cut on a straight grain (1) to within 3 mm. Smooth cutting of dress C.F (2) and cut on straight grain (2) to within 3 mm. Smooth cutting of dress C.B (2) and cut on straight grain (2) to within 3 mm. Front facing smoothly cut at the C.F (½) and cut on straight grain (½) Back facing smoothly cut at the C.B (½) and cut on straight grain (½) Smooth cutting of the dress lower edge front (1) and back (1) Sleeve binding cut on true bias | 3½ 2 4 4 1 1 2 1 |  |
|  |  | 18½ |  |
| 3. (a) (b) (c) (d) (e) | OVERLAID SEAM (if not overlaid award zero mark) Yoke placed over dress on the R.S (½) with raw edge of overlap well tucked under (½) Stitching line straight (1) and close to the fold (1) Correct width of neatened seam (½) 1 cm (± 2 mm) and even (½) Overlaid seams turnings facing up (½) at the sleeve end. Yoke and dress flashing at the seam. | 1 2 1 ½ 1 |  |
|  |  | 5½ |  |
| 4. (a) (b) (c) (d) | LOOP STITHCES (if not loop stitches award zero mark) (a – d) Evenness of stitches Correct tension of the stitches Uniform length of stitches Density of stitchery | 1 1 1 1 | 4 |
|  |  | 4 |  |
| 5. | SHOULDER SEAM – DOUBLE STITCHED [If not double stitched award zero mark (0)] |  |  |
| (a) (b) (c) (d) (e) (f) | First row of stitchery made with straight stitches Raw edges well tucked under Second line of stitchery made with straight stitches Evenness of seam (1) and correct size (1) 0.6 cm (± 2 mm) Flatness of seam on R.S (1) and on W.S (1) Fell facing back of dress | 1 2 1 2 2 1 |  |
|  |  | 9 |  |
| 6. (a) (b) (c) (d) (e) | SIDE SEAM [If not open seam award zero mark] (0) Seam joined with straight stitchery Well neatened raw edges (1 x 2) Evenness of seam allowance Correct size of seam allowance 2 cm (± 2 mm) Seam pressed open (1) and flat at armhole (½) R.S and (½) W.S | 1 2 2 2 2 |  |
|  |  | 9 |  |
| 7. (a) (b) (c) (d) | INVERTED PLEAT [If not inverted award zero mark (0) for (a)] Both folds facing away from each other on RS (1) and meeting on WS (1) to within 2 mm Correct width of inverted pleat 2 cm on R.S (1 x 2) within (±2 mm) at sleeve hem Pleat firmly fixed at sleeve hem (1) and well pressed (1) Pleat flat at sleeve hem | 2 2 2 1 |  |
|  |  | 7 |  |
| 8. (a) (b) (c) (d) (e) (f) (g) (h) (i) (j) (k) | SLEEVES Good hang Gathers well distributed (1) at the crown (1) Under arm seam joined with straight stitchery Armhole seam made with straight stitchery Notches matched at the armhole seam (½ x 2) Binding joined before attaching Binding attached using straight stitchery Binding seam and under arm seam flashing to within 2 mm Correct width of binding (1) i.e. (1 cm ± 2mm) and even in width (1) Underarm seam meeting side seam to within 2 mm Flatness of binding | 1 2 1 2 1 1 ½ ½ 2 1 1 |  |
|  |  | 13 |  |
| 9. (a) (b) (c) (d) (e) | SLIP HEMMING [If no slip hemming award zero mark (0)] Raw edge well tucked under Stitches worked by picking from the stitching line Correct tension Slip hemming with pleat concealed into binding Neatness of stitchery | 1 ½ 1 ½ 1 |  |
|  |  | 4 |  |
| 10. (a) (b) (c) (d) (e) (f) (g) (h) | NECK FACING Back and front facings joined with straight stitchery (½) seam trimmed (½) pressed open (½) and unneatened (½) Facing attached to waistline with smooth stitchery Seam trimmed (1) snipped (1) and understitched (1) Facing seam and shoulder seam meeting to within 2 mm at neckline Free edge of facing well neatened (2) and held down (½) at seamline (½) CF of blouse and CF of facing matching to within 2 mm CB of blouse and CB of facing matching to within 2 mm Flatness of neck facing | 2 2 3 ½ 3 ½ ½ 2 |  |
|  | Sub-total | 13½ |  |
|  | TOTAL MARKS | 90 |  |
|  | FINAL MARKS = ACTUAL MARKS 2 | 45 |  |