NAME: ………………………………………….	INDEX NO: ……………………………….

SIGNATURE: …………………………………	DATE : …………………………………..

SIGNATURE: ……………………………

TEACHER.CO.KE SERIES 37

232 / 3

PHYSICS

PAPER 3

(PRACTICAL)

TIME: 2 ½ HOURS

INSTRUCTIONS TO CANDIDATES

- Write your Name and Index Number in the spaces provided above.

- Sign and write the date of Examination in the spaces provided above.

- Answer all questions in the spaces provided.

- You are supposed to spend the first 15 minutes of the 2½ hours allowed for this paper reading the whole paper carefully before commencing your work.

- Marks will be given for clear records of observations actually made, their suitability, accuracy and the use made of them.

- Candidates are advised to record their observations as soon as they are made.

- All working must be clearly shown where necessary.

- Mathematical tables and silent electronic calculators may be used in calculations.

FOR EXAMINER’S USE ONLY

| Question | Maximum score | Candidate’s score |
| --- | --- | --- |
| 1 | 20 |  |
| 2 | 20 |  |
| TOTAL | 40 |  |

- You are provided with the following:-

- Two dry cells.

- Nichrome wire 100cm on a mm scale.

- An ammeter.

- Cell holder.

- Voltmeter.

- Connecting wires with crocodile clips.

- Switch.

Proceed as follows:

    - Connect the circuit as shown in the diagram.

    - Connect the ends A and C where AC is the length L of the Nichrome wire across the terminals as shown.  Close the switch and measure both current I and potential difference (p.d) across the wire AC when L = 100cm.

Current I =										(1mk)

p.d. V = 										(1mk)

    - Measure the E.m.f. of the cells, E.

E =											(1mk)

    - Reduce the length L (AC) to the lengths shown in the table below.  In each case record the current, I, and the corresponding p.d.				(7mks)

| Length L (cm) | 100 | 70 | 60 | 50 | 40 | 20 |
| --- | --- | --- | --- | --- | --- | --- |
| I (A) |  |  |  |  |  |  |
| P.d (V) |  |  |  |  |  |  |
| E – V (v) |  |  |  |  |  |  |

    - Plot a graph of E – V against (A) on x-axis in the grid provided.		(5mks)

    - Determine the slope of the graph.						(3mks)

    - Given that E = V + Ir, determine the internal resistance, r, of each cell. 	(2mks)

- You are provided with the following apparatus:-

- A metre rule.

- One stop watch. one stand, clamp and boss.

- One spring.

- Two pieces of wood.

- A beam balance or electronic balance (to be shared)

- One mass labeled M.

Proceed as follows:

        - Hang the spring vertically by clamping one end as shown in figure 1. (The small pieces of wood to clamp the spring).

        - Measure the length, Lo, of the unloaded spring, and record below.

Lo ___________________________mm					(½ mk)

        - Hang the mass M given from the lower end of the spring.  Measure the length, L1 of the loaded spring.

L1 = _________________________ mm					(½ mk)

        - Find the value of L1 – Lo in centimeters

L = L1 – Lo _____________________ cm					(1mk)

        - Using the balance given, find the mass of the object M.

Mass of M = __________________ g						(1mk)

        - Hang the mass M from the lower end of the spring.  Displace it by a small vertical distance and release so that the spring makes vertical oscillations.

Measure and record, time for the number of oscillations given in the table below.

| Oscillations, N | 5 | 7 | 10 | 13 | 15 | 18 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Time in seconds, t (s) |  |  |  |  |  |  |  |
| (N + 10t) (s) 10 |  |  |  |  |  |  |  |
| (N + 10t)2 (s2) 19 |  |  |  |  |  |  |  |

Complete the table above.								(7mks)

        - On the grid provided, plot a graph of   N + 10t 2 (y-axis) against N.		(5mks)

10

        - (i)  Determine the slope S, of the graph at N = 16.				(3mks)

(ii)  Find the constant k, given that:

K = MS									(2mks)

13L