END TERM 3 2026

GRADE 10

ELECTRICITY

SUMMATIVE ASSESSMENT

TIME: 2 HOURS 30 MINUTES

| Name: | ____________________________________________ |
| --- | --- |
| Assessment Number: | ____________________________________________ |
| Class / Stream: | ____________________________________________ |
| Date: | ____________________________________________ |

INSTRUCTIONS TO CANDIDATES

Write your name and assessment details in the spaces provided.

Read all questions carefully before answering.

Answer ALL questions.

Show all working where calculations are required.

Use appropriate units in calculations and standard electrical symbols in diagrams.

Label diagrams clearly where required.

Write answers in the spaces provided where applicable.

Use clear and legible handwriting.

Check that all pages and questions are present.

EXAMINER'S USE ONLY

| Section | Question Numbers | Maximum Score | Candidate's Score |
| --- | --- | --- | --- |
| A | 1–10 | 20 |  |
| B | 11–15 | 30 |  |
| C | 16–18 | 30 |  |
| D | 19–20 | 20 |  |
| TOTAL |  | 100 |  |

SECTION A — FOUNDATIONAL STRUCTURED QUESTIONS

Answer all questions in this section.

1. State two reasons why knowledge of electrical safety is important when carrying out electrical tasks. (2 marks)

________________________________________________________________________________
________________________________________________________________________________

2. Distinguish between a conductor and an insulator, giving one example of each. (2 marks)

________________________________________________________________________________
________________________________________________________________________________

3. Differentiate between a primary cell and a secondary cell. (2 marks)

________________________________________________________________________________
________________________________________________________________________________

4. State the principle on which a capacitor operates and name one application of a capacitor. (2 marks)

________________________________________________________________________________
________________________________________________________________________________

5. Name two types of pictorial drawing used in engineering representation. (2 marks)

________________________________________________________________________________
________________________________________________________________________________

6. State two magnetic properties of materials that are useful in electrical technology. (2 marks)

________________________________________________________________________________
________________________________________________________________________________

7. State two factors that affect the strength of an electromagnet. (2 marks)

________________________________________________________________________________
________________________________________________________________________________

8. What is meant by energy storage in an inductor? State one application of an inductor. (2 marks)

________________________________________________________________________________
________________________________________________________________________________

9. Name two stages or components involved between electrical generation and distribution to consumers. (2 marks)

________________________________________________________________________________
________________________________________________________________________________

10. State two items of equipment found at a consumer's power intake point. (2 marks)

________________________________________________________________________________
________________________________________________________________________________

SECTION B — APPLICATION, DATA, DIAGRAMS AND PROBLEM SOLVING

Answer all questions in this section.

11. A 12 V d.c. supply is connected to two resistors of 2 Ω and 4 Ω in series. (8 marks)

(a) Draw the circuit represented by the description, using standard electrical symbols. (3 marks)

DIAGRAM / WORKING AREA

(b) Calculate the total resistance of the circuit. (2 marks)

________________________________________________________________________________
________________________________________________________________________________

(c) Calculate the current flowing through the circuit. (2 marks)

________________________________________________________________________________
________________________________________________________________________________
________________________________________________________________________________

(d) State the expected reading of the voltmeter. (1 marks)

________________________________________________________________________________

12. A learner has two 1.5 V cells and a rechargeable secondary battery available for a low-voltage electrical application. (6 marks)

(a) Draw the arrangement of the two 1.5 V cells when connected in series and state the combined voltage. (3 marks)

DIAGRAM / WORKING AREA

(b) State three precautions for the safe maintenance or charging of rechargeable batteries. (3 marks)

________________________________________________________________________________
________________________________________________________________________________
________________________________________________________________________________

13. Two capacitors of capacitance 2 μF and 4 μF are connected in parallel across a 12 V supply. (6 marks)

(a) Calculate the total capacitance of the combination. (2 marks)

________________________________________________________________________________
________________________________________________________________________________

(b) State the voltage across each capacitor. (1 marks)

________________________________________________________________________________

(c) Explain why capacitors can be useful in electrical control circuits such as timer circuits. (3 marks)

________________________________________________________________________________
________________________________________________________________________________
________________________________________________________________________________
________________________________________________________________________________

14. A rectangular block is to be represented pictorially for an engineering drawing. (4 marks)

(a) Produce an isometric pictorial sketch of the block shown by the supplied diagram. (3 marks)

DIAGRAM / WORKING AREA

(b) State one advantage of pictorial drawings in engineering design. (1 marks)

________________________________________________________________________________

15. A current-carrying conductor and a solenoid are used to demonstrate electromagnetism. (6 marks)

(a) Draw the magnetic field pattern around a straight current-carrying conductor and indicate its direction. (3 marks)

DIAGRAM / WORKING AREA

(b) State three applications of electromagnets in electrical technology. (3 marks)

________________________________________________________________________________
________________________________________________________________________________
________________________________________________________________________________

SECTION C — EXTENDED STRUCTURED APPLICATION

Answer all questions in this section.

16. An electrical device uses inductors in its circuit. The designer must select and connect inductors appropriately. (10 marks)

(a) Explain how an inductor stores energy. (2 marks)

________________________________________________________________________________
________________________________________________________________________________

(b) Two inductors of 3 H and 6 H are connected in series. Calculate their equivalent inductance. (3 marks)

________________________________________________________________________________
________________________________________________________________________________
________________________________________________________________________________
________________________________________________________________________________

(c) State two characteristics of an inductor that should be considered when selecting one for a d.c. circuit. (2 marks)

________________________________________________________________________________
________________________________________________________________________________

(d) Explain one practical application of an inductor in an electrical or electronic appliance. (3 marks)

________________________________________________________________________________
________________________________________________________________________________
________________________________________________________________________________
________________________________________________________________________________

17. A rural area is supplied through an electrical power grid. (10 marks)

(a) Draw a labelled block diagram showing the main stages from generation to distribution to consumers. (5 marks)

DIAGRAM / WORKING AREA

(b) Explain why high-voltage transmission is used in the power grid. (2 marks)

________________________________________________________________________________
________________________________________________________________________________

(c) State three reasons why a reliable national grid is important to socio-economic development. (3 marks)

________________________________________________________________________________
________________________________________________________________________________
________________________________________________________________________________

18. An electrician is preparing to complete an electrical installation in a domestic building. (10 marks)

(a) Identify four items of equipment that may be found at the consumer's intake point. (4 marks)

________________________________________________________________________________
________________________________________________________________________________
________________________________________________________________________________
________________________________________________________________________________

(b) State the correct sequence in which the main control/protection equipment is connected at the consumer's intake point. (3 marks)

________________________________________________________________________________
________________________________________________________________________________
________________________________________________________________________________

(c) State three checks or considerations an electrician should make before commissioning a final circuit. (3 marks)

________________________________________________________________________________
________________________________________________________________________________
________________________________________________________________________________

SECTION D — ELECTRONICS AND INTEGRATED PROBLEM SOLVING

Answer all questions in this section.

19. Semiconductor materials are used to construct electronic devices. (10 marks)

(a) Explain the difference between intrinsic and extrinsic semiconductors. (3 marks)

________________________________________________________________________________
________________________________________________________________________________
________________________________________________________________________________

(b) Explain how doping produces P-type and N-type semiconductor materials. (3 marks)

________________________________________________________________________________
________________________________________________________________________________
________________________________________________________________________________
________________________________________________________________________________

(c) Use a labelled sketch to show the formation of a PN junction. (2 marks)

DIAGRAM / WORKING AREA

(d) State two reasons semiconductor materials are important in electronic engineering. (2 marks)

________________________________________________________________________________
________________________________________________________________________________

20. A technician is testing a diode and a transistor in an electronic switching circuit. (10 marks)

(a) Describe the operation of a semiconductor diode under forward bias and reverse bias. (4 marks)

________________________________________________________________________________
________________________________________________________________________________
________________________________________________________________________________
________________________________________________________________________________

(b) State two applications of diodes in electronic systems. (2 marks)

________________________________________________________________________________
________________________________________________________________________________

(c) A transistor is used as a switch. Explain how changing its input condition can control current in the output circuit. (3 marks)

________________________________________________________________________________
________________________________________________________________________________
________________________________________________________________________________
________________________________________________________________________________

(d) State one safety or troubleshooting precaution when testing electronic components in a circuit. (1 marks)

________________________________________________________________________________